# Practical session 7: Commit the redesign and start retesting

Mentor Teaching · Teacher directions · 15 October 2026

# Plan and prerequisites

45–60 minutes; allow continuation and complete the first phase beforehand. Planning estimate; adjust to the class.

Accurate decimal °C/time reading; calculate change; measure 150 mL; follow the matched-pair and probe-position procedure. Carry forward the preceding session evidence. The adult must check the current complete operational guide, equipment and room/water conditions before handling. Three retained first-comparison records and the original construction version are required.

# Print in this order

1. Practical redesign: commit the plan

2. Practical retest: instrument checks and trial 1

Give each independent or new-evidence file only at the stage described below. Keep answers with the teacher.

# Teach and demonstrate

Compare the completed F1–F3 records before designing. Retain the original version. The redesign is compared with the original one-sheet sleeve, not the bare cup. Check the prediction, changed feature, two-sheet/30 cm tape limits, footprint/height limits and uncovered base/top/probe slot. Teacher checks safe fit and insertion depth before water use.

# Learner work and evidence

Retain the learner’s plan and prediction before testing. Run R1 using a fresh matched water pair and the printed retest order: original sleeve left/A, redesigned sleeve right/B. Record all standard conditions, 0/5/10 readings, actual times and room air. Save the pre-phase instrument check and plan for session 8.

# Next decision and continuation

If any F trial is unfinished, continue that phase first. Do not count a proposed redesign as physically tested. At a break, drain/clean/store dry labelled versions; retain the R1 log. The next session completes R2–R3 with fresh pairs. Never substitute the fictional retest values for missing empirical readings.

# Matching complete teacher key

Use the current complete operational guide for redesign, retest reference/order, phase checks and redesign keys. Keep the later reasoning check unseen.

Open the matching complete teacher guide

# Access and interpretation

Offer oral, dictated, pointing or labelled responses when they show the same intended reasoning. Keep each task’s quantities, conditions and sources available. Allow extra response paper. A supplied-data explanation does not establish actual construction or testing.